PURCHASE SPECIFICATIONS FOR A CONTROL & MONITORING UNIT

| Model | UR-201 |
| --- | --- |
| Type | Control & Monitoring Unit (CMU) |

- Control & Monitoring Unit Specification

Control & Monitoring Unit (CMU) provides the User with the ability to remotely control and monitor the Solar Airfield Lighting System. It provides remote control over the lights from the ATC tower, pilot radio, and over a cellphone.

Control & Monitoring Unit shall allow for:

- Remote control and monitoring for Solar Airfield Lighting System and other radio-controlled equipment,

- 3-way remote control over airfield lighting:

  - From ATC, manually – by using knobs located on the control unit front panel,

  - Via Air-Band Radio: frequency range 868 MHz (optionally: 2.4 GHz or 915 MHz),

  - Via GSM Phone (voice call or SMS);

- Independent control (activation and change of brilliancy) over a minimum of 3 separate groups of airfield lighting (for example, Runway + Approach, Taxiway, PAPI),

- In case of Wireless signal failure, the light shall allow for manual activation using the Emergency On/Off Button,

- Automatic Alarm SMS: in case of the light’s critical failure (e.g. lack of wireless control over the light or low battery), the User will automatically receive an alarming SMS informing them about the critical failure.

Control & Monitoring Unit shall offer the following features:

- Rechargeable back-up battery providing a minimum of 24 hrs of autonomy (with all functions operating at all times),

- Power supply via 90-240 VAC,

- Detachable antennas,

- Externally mounted (on the high mast) antenna for optimal radio range,

- Immediate and simultaneous on/off control of the airfield lights,

- Grouping entire airfield lighting/equipment into at least 3 major groups

- (controlled separately),

- Control of the entire lighting system or separate groups of lights,

- Selecting preferable operating modes such as dusk-till-dawn, remote (allows for remote activation of lights) or flash,

- Ability to remotely activate the system via VHF, GSM and manual switches,

- Minimum 3-step intensity level of a group of lights,

- Setting a timer for remotely activated lights,

- Built-in circuit breaker protecting against short circuit/overload,

- Built-in antennas, lighting strike arresters,

- Built-in transportation fuse,

- Built-in USB port / HDMI Port.

2.0. Wireless Control Range

The system shall be capable of reliable wireless control up to a 1,5 kilometers line-of-sight distance between the controller and any light.