PURCHASE SPECIFICATIONS FOR AN AIRFIELD CONTROL & MONITORING SYSTEM FOR S4GA RADIO CONTROLLED EQUIPMENT

| Model | ALCMS |
| --- | --- |
| Type | Basic |

- Control & Monitoring System

The control and monitoring system shall consist of two elements:

- HMI Workstation with Airfield Lighting Control and Monitoring System

- UR-201 Control & Monitoring Unit with built-in UPS (see UR-201 Control & Monitoring Unit specification for more details)

- Separate UPS for HMI

2.0. Airfield Lighting Control and Monitoring System Specification

ALCMS  (airfield lighting control and monitoring system) is a software-based visual interface allowing users to control & monitor the entire airfield lighting system using a touchscreen monitor.

ALCMS shall have password protection for User and Administrator modes.

ALCMS shall enable individual control & monitoring capability for individual light units:

At least the following parameters of every light shall be monitored:

- Battery level in %,

- Operating time left on battery only (per light/equipment / per group / per system,

- CO2 savings calculation by using solar-powered equipment,

- Solar Panel Charging efficiency in %,

- Equipment internal temperature in Celsius or Fahrenheit,

- Charging source (wire/solar),

- Online connection status,

- Total equipment up-time,

- Total lights illumination time,

- Battery preventive maintenance scheduling (days and battery cycles used).

ALCMS shall enable individual control & monitoring capability for every Solar Engine:

At least the following parameters of every light shall be monitored:

- Battery level in %,

- Operating time left on battery only (per light/equipment / per group / per system,

- Online connection status,

ALCMS shall offer the following features:

- Grouping entire airfield lighting into a maximum of 6 individual groups,

- Control of the entire lighting system or separate groups of lights,

- Minimum 5-step intensity level of a group of lights (optionally up to 8-steps),

- Selecting preferable operating modes such as dusk-till-dawn, NVG (optional, NVG=infra-red) or remote activation mode (via GSM, via VHF radio),

- Enabling or disabling remote activation (via GSM or VHF radio),

- Setting timer for lights activated remote means of activation (VHF, GSM)

- Individual light status – providing the user with real-time monitoring of each individual lights’ parameters (battery level, charging status, online connection, solar panel charging level, maintenance requirements, etc.),

- Monitoring of the control unit (status feedback, back-up battery level, power connection),

- Alarm log that stores every critical failure of the system,

- Colors of interface indicating general status of lights (red: critical fault, amber: requires attention).

3.0. Hardware Specification

3.1. Control & Monitoring Unit Specification

Control & Monitoring Unit shall allow for:

- Remote control and monitoring for Solar Airfield Lighting System and other S4GA radio-controlled equipment,

- Independent control (activation and change of brilliancy) over a minimum of 6 separate groups of airfield lighting (for example, Runway, Approach, Taxiway, PAPI APP1, PAPI APP2, etc.),

- In case of Wireless signal failure, the light shall allow for manual activation using the Emergency On/Off Button,

Control & Monitoring Unit shall offer the following features:

- Rechargeable back-up battery providing a minimum of 24 hrs of autonomy (with all functions operating at all times),

- Power via 90-240 VAC,

- Detachable antennas, externally mounted (on the high mast) antenna for optimal radio range,

- Immediate and simultaneous on/off control of the airfield lights,

- Grouping entire airfield lighting/equipment into at least 3 major groups (controlled separately),

- Control of the entire lighting system or separate groups of lights,

- Selecting preferable operating modes such as dusk-till-dawn, remote (allows for remote activation of lights) or flash,

- Ability to remotely activate the system via VHF, GSM and manual switches,

- Minimum 3-step intensity level of a group of lights,

- Setting a timer for remotely activated lights,

- Built-in circuit breaker protecting against short circuit/overload,

- Built-in antennas, lighting strike arresters,

- Built-in USB port / HDMI Port.

3.2. HMI Workstation Specifications

Processor - Intel Core i5-9400 (6 cores, from 2.90 GHz to 4.10 GHz, 9 MB cache)

Chipset - Intel H370

RAM memory - 16 GB (DDR4 DIMM, 2666 MHz)

Graphics card – supporting FullHD technology, Intel UHD Graphics 630

M.2 SSD - 256 GB

Sound - Integrated sound card

Connectivity - Wi-Fi 4 (802.11 b / g / n), LAN 10/100/1000 Mbps

Connectors - USB 3.0 - x2, USB 3.0 - x2, Audio input / output - 3 RJ-45 (LAN) - 1 VGA (D- sub ) – 1 HDMI – 1 AC-in (power input ) - 1.

3.3 Touch Screen Monitor Specification

Border color - black

Active display area - 698.4 mm (H) x 392.9 mm (V) or 27.49 inches (H) x 15.47 inches (V)

Dimensions - IR model: 762.8 mm (L) X 457.3 mm (H) X 61.6 mm (D) Or 30.03 inches (L) X 18.00 inches (H) X 2.43 inches (D)

LCD technology - TFT LCD active matrix

Mounting options   - VESA MOUNT PER MIS-F, 400,400,6MM

Input/output ports - Input: power input, USB type B (on touch), VGA, 2x HDMI, GPIO, DisplayPort, Audio Line in Outputs: Audio headphone output, RJ45 (for optional OSD remote control)

Resolution - 1920x1080 at 60Hz

Aspect ratio - 16: 9

3.4 Workstation UPS Specification

Topology - Line-Interactive

Apparent power - 1500 VA

Effective power - 1350W

Input voltage - 0 - 300 V

Output voltage shape - Sinusoidal

Output sockets - IEC - 8  RJ-11 (in / out) RJ-45 (in / out)

Switching time - 2 - 6 ms

Average charging time - 4 hours

Communication interface   - RS232, USB

Work signaling - LCD display, LED diodes

Enclosure type - Tower, Rack

Additional information - Automatic Voltage Regulation (AVR), Function Emergency Power Off EPO (Emergency Power Off)

3.5 Mouse and keyboard set Specification

Connectivity -  wireless

Interface - USB

Numeric keys - Yes

Color - black

Mouse included - Yes

Mouse sensor - optical

Mouse profile - universal

Mouse resolution - 1000 DPI