PURCHASE SPECIFICATIONS FOR THE HANDHELD CONTROLLER

(FOR RADIO CONTROLLED EQUIPMENT)

| Model | UR-101 |
| --- | --- |
| Type | Handheld Controller for Radio Controlled Equipment |

##### General Overview

Handheld Controller provides the User with the ability to remotely control airfield and heliport lighting systems.

Handheld Controller shall allow for:

- Remote control of airfield and heliport lighting systems and other radio-controlled equipment,

- Independent control (activation and change of brilliancy) over a minimum of 5 separate groups of lighting,

Handheld Controller shall offer the following features:

- Grouping entire lighting/equipment into at least 5 major groups (controlled separately),

- Minimum 3-step intensity level of a group of lights,

- Control of the entire lighting system or separate groups of lights,

- Selecting operating modes such as dusk-till-dawn, or NVG (on the level of the entire system or separate groups),

- Operating frequency: 868 MHz (optionally: 2.4 GHz or 915 MHz),

- Rechargeable back-up battery providing a minimum of 48 hrs of autonomy (with all functions operating at all times),

- Battery charging and power supply shall be via a 110 - 230 VAC Charger,

- Detachable antenna.

Wireless Control Range

The system shall be capable of reliable wireless control over distances greater than 10 km (relayed) in line-of-sight conditions between the controller and connected lighting equipment.