PURCHASE SPECIFICATIONS FOR A SOLAR LIGHT

| Model | SP-501SH |
| --- | --- |
| Type | High Intensity Solar Runway Edge Light |
| Optics Type | Bidirectional type with omni-directional component |
| Optics Configuration | White / White / Yellow |

##### General Overview

Each light shall consist of or be capable of:

- LED optics,

- Non-corrosive polycarbonate casing,

- Built-in microcomputer with integrated plug-in radio-transceiver (replaceable),

- Externally mounted (replaceable) and tilted solar panel for optimal energy collection,

- Dual-function power ports allowing to energize the light:

  - via solar panel and,

  - electrical grid,

- Two independent built-in batteries (connected in parallel),

- External (replaceable) antenna for wireless control & monitoring,

- Wireless network should be a mesh (not point-to-multipoint) protocol and operate in a non-licensed frequency band such as 868 MHz (optionally 915 MHz or 2,4 GHz),

- Each Light should be equipped with an emergency On/Off button.

The entire light shall be delivered complete and ready to install. It shall not require assembly except for the solar panel and mounting accessories.

1.0 Physical and Mechanical Parameters

- The light shall have a solar panel mounted externally and connected to the light via a power port,

- All batteries and electronics shall be contained within the light,

- The maximum height of the solar light (excl. antenna) shall not be more than 360 mm,

- The maximum weight of the solar light shall not be more than 15 kilograms,

- Solar-powered airfield light (fully assembled) shall be tested against jet blast and wind load of a minimum speed of 240 kph. A compliance report of actual wind tunnel testing (not computer simulations) shall be provided,

- The body of the light shall be polycarbonate,

- The light’s dome shall be borosilicate glass,

- The light should be equipped with a waterproof pressure stabilizing valve,

- Body of the light shall have an Ingress Protection rating of min. IP68. Compliance shall be confirmed by a test report issued by a third-party laboratory or institute,

- Light shall have an Impact Rating of not less than IK10. Compliance shall be confirmed by a test report issued by a third-party laboratory or institute,

- The light’s dome shall be replaceable on site in case of damage,

- The optical LED head shall be replaceable. The manufacturer shall offer complete optics replacement kits, including required tools to perform the replacement,

- The batteries shall be of standard type available from the local battery stores (worldwide battery standard),

- All types of lights (including runway, taxiway and obstruction) shall be equipped with the same type and capacity of battery.

1.1 Mounting of the lighting unit

- The light shall be capable of being mounted on all types of surfaces, including grass and concrete,

- The mounting shall include a frangible coupling,

- Mounting components should be manufactured of high-quality material with high metal corrosion resistance (ie. stainless steel).

1.2 Light Capabilities

- The light shall be capable of at least 99 x different brilliancy levels,

- The light shall be capable of operation during the day if required,

- The light shall be equipped with a manual on/off button to allow for light activation when it is operated without wireless control,

- The lights shall be equipped with an LED indicator showing: [1] battery level and also [2] whether the solar panel or the charging station is charging the battery.

By switching the manual on/off button on any light installed on the airfield, the rest of the lights located within radio range shall be remotely activated in less than 1 second.

2.0 Operational Parameters

2.1 Optical Performance

LEDs must have a lifespan of at least 100.000 hours.

The light shall provide optical performance meeting or exceeding the following specifications:

- a minimum average White light intensity of no less than 10,000 candela (cd) in accordance with ICAO Annex 14th, Volume I, 9th Edition dated July 2022, clause 5.3.9.10, Figure A2-10 & clause 5.3.1.11, Figure A2-11, Appendix 1, Figure A1-1b,

- a minimum average Yellow light intensity of no less than 4,000 candela (cd) in accordance with ICAO Annex 14th, Volume I, clause 5.3.9.10, Figure A2-10 & clause 5.3.1.11, Figure A2-11, Appendix 1, Figure A1-1b.

All values are tested and certified by a third-party laboratory.

Optionally, optics can be additionally equipped with Infra-Red LEDs to enable covert operations using NVG goggles.

2.2 Solar Panel

- The lighting unit shall use a solar panel installed separately from the light unit body,

- The entire solar panel shall face one geographical direction for optimal energy collection,

- Solar panel shall be mounted at a circa 33 degrees tilt,

- The entire solar panel shall have a minimum nominal power output of 25 watts,

- Solar panel mounting shall allow for replacing the entire solar panel in case of breakage,

- The mounting system shall allow the solar panel to be directed towards the optimal sun exposure for maximum energy collection.

2.3 Energy storage

- All models of the light shall be equipped with nominal 12-volt battery systems,

- The lights shall be equipped with two batteries; each battery shall have a minimum power capacity of 108 Wh. Total capacity of batteries shall not be less than 216 Wh,

- Failure of one of two batteries shall not prevent the light from operating – hence the light shall be able to operate only on one battery,

- The batteries shall be of one of the following types: Valve-Regulated Lead-Acid (VRLA), Lead-Acid Cyclon, Lithium-Ion (Li-ion), or Lithium Iron Phosphate (LiFePO4),

- The batteries shall be replaceable. Replacement shall require no special tools and shall take no longer than 15 minutes per light,

- Manufacturers offering lights energized by self-designed battery packs shall offer a standard, globally available battery,

- Light unit shall be equipped with a built-in battery status indicator,

- The battery operating temperature range published by the manufacturer shall be at least from -20 to +50 °C,

- The light’s operation in autonomous mode shall be designed to ensure both sufficient battery lifespan and continuous performance at minimum intensity levels, as outlined below:

| Battery type | Battery lifespan, cycles | Operation time at minimum intensity level, hours |
| --- | --- | --- |
| Lead Acid | 1,200 | 180 |
| Li-ion | 3,000 | 340 |
| LiFePO4 | 3,000 | 240 |
| Lead Acid Cyclon | 300 | 100 |

2.4 Power

The light shall be capable of drawing power from its internal battery.

The battery shall be capable of being charged by four alternative methods:

- via solar panel,

- via electrical cable (24VDC),

- via back-up charging station,

- via drop-in charging port.

When connected to an external electric grid (24VDC) and in case of battery failure, the lighting unit shall still be able to continue operation.

2.5 Electronics

The light’s control system shall have:

- Temperature-compensated Maximum Power Tracking battery charging,

- Low-voltage cut-off to prevent over-discharge of the battery system,

- Temperature sensor,

- Battery capacity temperature compensation logic (to show temperature-calibrated battery capacity).

The light and controller shall be capable of receiving firmware upgrades.

3.0 Wireless Specifications

- Light shall be wirelessly controlled,

- Wireless communication should use the mesh protocol (manufacturers using the point-to-multipoint radio protocol shall offer the mesh protocol).

3.1 Wireless Signal

- The light’s antenna shall be detachable/replaceable,

- The light should be equipped with an external antenna to maximize radio range,

- There shall be no limit to the number of lights the controller can communicate with, provided they are within the required radio range,

- The wireless system shall communicate using non-licensed 868 MHz (optionally 915 MHz or 2,4 GHz) radio frequency with a power output of no more than 20 mW,

- The frequency band shall not require special approval and shall not be designated for other types of communication (e.g., GSM carriers: 900 / 1800 MHz),

- The system shall be capable of normal operation in the presence of RF activity typical for an airport environment.

4.0 Quality Assurance

Excluding the batteries, the system, including solar panels, LEDs, optics, electronics, mechanicals and associated components, shall carry a minimum two-year warranty. The batteries shall be guaranteed for 1 year.

The light shall be manufactured by an ISO 9001:2008 certified manufacturing facility.

5.0 Turn-Key Operation

The light shall be ready for installation upon delivery. Assembly consists only of threading the antenna onto the light, activating the light through a single button-press on the light, and attaching it to the mounting accessories and solar panel.