PURCHASE SPECIFICATIONS FOR A POWER BANK

| Model | PB-3000 |
| --- | --- |
| Type | Power Bank |
| Power Supply | 110-230V |

- General Overview

Power Bank shall be designed to power airfield lighting equipment, such as portable LED PAPI systems and compatible with LHA-type PAPI units. A single Power Bank can power 4x PAPI projectors at the same time.

Power Bank shall consist of the following components:

- 1 x Inverter-Charger,

- 1 x Air filter,

- 2 x batteries 12V / 125Ah (total capacity of 3 000 Wh),

- 4 x Output sockets 110-230V 16A (military standard sockets),

- 1 x Input socket 110-230V 20A (military standard socket),

- 1 x Emergency Shutdown Switch,

- 1 x Ventilation panel.

Shall be equipped with Victron Multiplus C12/1600/17 Inverter-Charger.

Shall have Ingress Protection rating of minimum IP-67.

Shall be able to operate in the temperature of -40 to +55 ºC (-40 to 131 ºF) or better.

2.0 PAPI Autonomy Provided by Power Bank

Power bank shall provide up to 10 hours of LED PAPI (4x LHA system) autonomy.

Power Bank capacity shall not be less than 3 000 Wh.

3.0 Power Supply

Input power supply shall be 110-230V 20A.

Output power supply shall be 4x 110-230V 16A.

4.0 Battery

Power bank shall include battery monitoring functionality. Battery status shall be available to the user for operational control.

Power bank shall be equipped with overcharging protection.

5.0 Casing

Power bank casing shall have the following features:

- Watertight, crushproof, and dustproof,

- Stainless steel hardware and padlock protectors,

- Automatic Pressure Equalization Valve,

- Open cell core with solid wall design,

- O-ring seal.