PURCHASE SPECIFICATIONS FOR AN UNINTERRUPTIBLE POWER SUPPLY UNIT

| Model | UPS-3000 |
| --- | --- |
| Type | Uninterruptible Power Supply Unit |
| Power Supply | 110-230V |

- General Overview

The Uninterruptible Power Supply Unit (UPS) is used to provide a power supply to the LED PAPI systems. 
A single UPS can power 4x PAPI projectors at the same time.

UPS shall consist of the following components:

- 1 x Inverter-Charger,

- 1 x Air filter,

- 2 x batteries 12V / 125Ah (total capacity of 3 000 Wh),

- 4 x Output sockets 110-230V 16A (military standard sockets),

- 1 x Input socket 110-230V 20A (military standard socket),

- 1 x Emergency Shutdown Switch,

- 1 x Ventilation panel.

Shall be equipped with Victron Multiplus C12/1600/17 Inverter-Charger.

Shall have Ingress Protection rating of minimum IP-67.

Shall be able to operate in the temperature of -40 to +55 ºC (-40 to 131 ºF) or better.

2.0 PAPI Autonomy Provided by UPS

UPS shall provide up to 10 hours of LED PAPI (4x LHA system) autonomy.

UPS capacity shall not be less than 3 000 Wh.

3.0 Power Supply

Input power supply shall be 110-230V 20A.

Output power supply shall be 4x 110-230V 16A.

4.0 Battery

UPS shall include battery monitoring functionality. Battery status shall be available to the user for operational control.

UPS shall be equipped with overcharging protection.

5.0 Casing

UPS casing shall have the following features:

- Watertight, crushproof, and dustproof,

- Stainless steel hardware and padlock protectors,

- Automatic Pressure Equalization Valve,

- Open cell core with solid wall design,

- O-ring seal.